Mapping GF(256) to GF(162)

For Hardware Inversion of GF(256)

rcgldr   3/9/07

There are 2 fields of GF(16) with primitive b = 1 x  +  0, or 2 hex. This document will only use GF(16) based on the polynomial x4 + x + 1 .

There are 16 fields of GF(256) with primitive a = 1 x  +  0, or 2 hex. This document will only use 2 of them based on the polynomials x8 + x4 + x3 + x2 + 1 and x8 + x7 + x2 + x1 + 1.

There are 64 fields of GF(162) with a = 1 x  +  0, or 10 hex, with GF(16) based on x4 + x + 1.

Only a few of the 64 fields can be used to map from any specific GF(256). The requirement for mapping is that math operations of a‘s in both GF(256) and GF(162) fields must be identical. For example, given any a and b, and defining  c and d as: ac = aa + ab and ad = aa ab , c and d will be the same for both GF(256) and GF(162) fields. When the two fields meet this requirement, then a field can be mapped to the other by multiplication by an 8 bit by 8 bit matrix and mapped back by with the inverse of the matrix.

For the GF(256) based on the polynomial  x8 + x4 + x3 + x2 + 1 (11D hex), there are 4 compatible GF(162) fields:  x2 + 2 x + 5, x2 + 3 x + 4, x2 + 4 x + 2, and x2 + 5 x + 3.

For the GF(256) based on the polynomial  x8 + x7 + x2 + x1 + 1 (187 hex), there are 4 compatible GF(162) fields:  x2 + 9 x + 2, x2 + b x + 5, x2 + d x + 4, and x2 + e x + 3.

After choosing compatible fields, mapping from one to the other is accomplished by matrix multiplication. The byte to be converted is treated as an 8 row by 1 bit wide matrix, with most significant bit at the top. For example naming the byte bits as new and old:

çnew7	ç		ç1 0 0 1 0 0 0 0ç		çold7	ç

çnew6	ç		ç1 1 1 1 0 1 0 0ç		çold6	ç

çnew5	ç		ç0 0 1 0 0 0 0 0ç		çold5	ç

çnew4	ç	=	ç1 0 1 0 1 0 1 0ç		çold4	ç

çnew3	ç		ç1 1 1 0 1 0 0 0ç		çold3	ç

çnew2	ç		ç0 1 0 0 0 0 0 0ç		çold2	ç

çnew1	ç		ç0 1 1 1 0 1 0 0ç		çold1	ç

çnew0	ç		ç0 0 1 0 0 0 0 1ç		çold0	ç

Using GF(256) based on the polynomial  x8 + x4 + x3 + x2 + 1 (11D hex), and GF(162) based on the polynomial  x2 + 4 x + 2 =  x2 + b2 x + b,  the following matrices are used for mapping:

GF(256) ® GF(162)      GF(162) ® GF(256)

1 0 0 1 0 0 0 0       0 1 0 0 0 0 1 0

1 1 1 1 0 1 0 0       0 0 0 0 0 1 0 0

0 0 1 0 0 0 0 0       0 0 1 0 0 0 0 0

1 0 1 0 1 0 1 0       1 1 0 0 0 0 1 0

1 1 1 0 1 0 0 0       0 1 1 0 1 1 1 0

0 1 0 0 0 0 0 0       1 1 1 0 0 1 0 0

0 1 1 1 0 1 0 0       0 0 0 1 1 1 0 0

0 0 1 0 0 0 0 1       0 0 1 0 0 0 0 1

Inversion of a GF(256) byte is done by mapping to GF(162) with left matrix. Let a1 represent upper 4 bits of mapped byte, and a0 represent lower 4 bits, so that mapped GF(162) byte can be represented as:  a1 x + a0 . Defining the inverse byte as:  b1 x + b0 , the equations are:

b1=a12a12+4a1a0+a02

b0=4a1+a02a12+4a1a0+a02

After calculating the b’s, map the byte back using the above right matrix.

For GF(256) based on the polynomial x8 + x7 + x2 + x1 + 1 (187 hex), and GF(162) field based on   x2 + 9 x + 2 =  x2 + b14 x + b,  the following matrices and equations are used for mapping:

GF(256) ® GF(162)      GF(162) ® GF(256)

1 0 0 1 1 1 0 0       1 1 0 0 0 0 1 0

1 1 0 1 1 0 0 0       1 1 0 0 1 1 1 0

0 0 1 1 1 0 0 0       0 1 1 0 1 1 0 0

1 1 1 1 1 1 1 0       1 0 1 0 0 1 1 0

1 0 1 1 0 0 0 0       1 1 1 0 1 0 1 0

0 1 1 1 0 0 0 0       0 0 0 0 1 1 1 0

1 1 0 0 0 1 0 0       0 0 1 1 0 0 1 0

0 0 1 1 1 0 0 1       0 0 1 0 0 0 0 1

b1=a12a12+9a1a0+a02

b0=9a1+a02a12+9a1a0+a02